Carbon footprint reduction in construction using geopolymer cement

Jebin Raj J, Anto Aakash M, Sanjana A

Abstract

In the production of ordinary Portland cement there are 8% (approx.) of carbon emission throughout the world due to the process of calcination of limestones and high temperature kiln operations. Using geopolymer cement it reduces the carbon footprint and utilizes less energy than OPC. The geopolymer cement is the byproduct of industrial waste like fly ash and ground granulated blast furnace slag (GGBS) it is activated by alkaline solution like sodium hydroxide and sodium silicate. In this project to reduce the carbon emission without compromising the strength and durability. Geopolymer cement achieves a good mechanical strength by using the industrial waste and avoiding the calcination. In the study of geopolymer bricks it is a good eco-friendly alternative material for traditional bricks. The bricks are manufactured using geopolymer binder. The CO2 emission is reduced by this method to maintain a sustainable construction.

Key words: geopolymer cement, ordinary Portland cement, carbon emission, geopolymer bricks

Introduction

In present construction industry, the increasing concern about environmental pollution and carbon emissions have lead to the search of alternative building materials. Ordinary Portland Cement (OPC), which is most widely used binder, contributes nearly 7 to 8% of the global CO₂ emission. This is due to the calcination of limestone and the high energy consumed during cement manufacturing. The demand for construction is increasing day by day and that also increase the use of cement, which make the problem more serious.

To overcome this issue we are focusing on more sustainable and low-carbon binders like geopolymer cement. It is produced by using industrial waste materials such as fly ash and Ground Granulated Blast Furnace Slag (GGBS), which are rich in silica and alumina. These materials are activated with alkaline solutions like sodium hydroxide and sodium silicate to form a cementitious binder without using limestone. This means that the CO₂ emission from clinker production is completely avoided.

Geopolymer cement not only helps in carbon reduction but also improve durability and chemical resistance of construction materials. In this project we will prepare eco-friendly bricks using geopolymer cement it act as a binder. The aim is to identify the strength and quality of these bricks and compare with traditional bricks. If it success, this method can be good solution for green construction.

Objectives

- Using geopolymer cement it reduce the carbon emission in construction

- To study the workability and setting behaviour of the bricks using geopolymer mix

- Geopolymer cement is made up of industrial waste like fly ash and GGBS it promotes the ecofriendly product

- To test the compressive strength of the geopolymer bricks at different curing condition

Methodology

1)Collect Raw Materials

Fly ash (Class F), GGBS, sodium hydroxide (NaOH), sodium silicate (Na₂SiO₃), sand, and water.

2) Prepare Alkaline Activator

Mix NaOH solution (e.g., 10M) with Na₂SiO₃ in a 2.5:1 ratio and allow the solution to rest for 24 hours.

3) Mixing

The fly ash and GGBS are combined in a desired ratio and also gradually add the alkaline solution and mix thoroughly. add sand if needed.

4) Brick Casting

The fresh mix is poured into standard brick moulds (230×110×75 mm) after that compact properly to remove air voids.

5)Curing

Curing At room temperature for 7 or 28 days (ambient curing). And the alternative method is bricks are placed in the oven at the temperature of (60°C to 80°C) for 24 or 48 hours

6)Testing

Test the bricks for compressive strength, water absorption, size and surface quality

7) Compare Results

Compare the performance of geopolymer bricks with conventional OPC bricks and Estimate potential CO₂ emission reduction

Implementation

In this study, the geopolymer bricks are prepared using industrial waste material like fly ash and GGBS. First, the raw materials such as fly ash, GGBS, sodium hydroxide (NaOH), sodium silicate (Na₂SiO₃), and sand was collected. Sodium hydroxide solution was prepared with 10 molar concentration and mixed with sodium silicate in the ratio of 2.5:1. The solution was kept for 24 hours before use.

Then, the fly ash and GGBS was mixed in dry condition. After that the alkaline activator solution was added slowly and mixed well. Sand was also added to improve the workability and shape of the brick. The mix was poured into standard moulds of brick size (230×110×75 mm) and compacted properly.

The bricks were cured under two different methods one batch was heat cured at 70°C for 24 hours, and another method was kept for ambient curing for 14 and 28 days. After curing the bricks were tested for compressive strength and water absorption.

This implementation shows that geopolymer cement can be successfully used in making bricks and also reduce the carbon emission compared to normal cement bricks.

Expected Outcomes

1)Bricks made by geopolymer cement are likely to demonstrate positive results in the compressive strength; it would be fairly or even better than the ordinary OPC bricks.

2) Fly ash and GGBS will also be employed to make good use of industrial waste, and this will curtail petroleum pollution as well as land fill issues.

3) Geopolymer cement will have very low carbon emission levels in production since it does not involve any limestone calcination.

4) The bricks can be observed superior durability and chemical resistance; particularly in aggressive environments.

5) The heat cured bricks are supposed to be stronger quicker than the ambient cured bricks and this will save construction time in the setting or pre-casting activity.

6) In general, the given research will most likely demonstrate that geopolymer bricks may become an eco-friendly and sustainable replacement of traditional clay and cement bricks in a construction industry of the future.

Advantages of Geopolymer Cement Bricks

1) Geopolymer bricks do not involve limestone burning, so it produce very less CO₂ compared to OPC bricks.

2) Materials like fly ash and GGBS, which are industrial waste are used, so it helps in waste management.

3)The bricks are strong and durable, suitable for structural applications.

4)Since it reduces emission and uses waste material it support green construction and sustainable development.

Limitations of Geopolymer Cement Bricks

1)The alkaline activators like sodium hydroxide and sodium silicate are little expensive compared to water used in OPC, which increase the cost.

2)NaOH is a strong chemical and need careful handling during mixing, which may not be safe in small sites.

3)Fly ash and GGBS are industrial waste and may not be available in all regions equally.

Applications of Geopolymer Cement Bricks

1) It can be used in residential, commercial, and public buildings as a good sustainable option instead of using clay or OPC bricks.

2) It is also ideal for walking paths and footpaths because of its good durability

3) Help to convert waste materials like fly ash, GGBS and other industrial waste into useful bricks for building purpose.

4) Work well in hot regions because it don’t transfer heat easily and gives better insulation inside buildings.

Budget estimation

| Item | Quantity | Rate(₹) | Cost(₹) |
| --- | --- | --- | --- |
| Raw materials (sand, fly ash, GGBS, NaOH, Na2SiO3) | 7000 to 8000 bricks | - | 90,000 |
| Steel brick moulds (heavy duty) | 10 to 12 | 3,000 | 36,000 |
| Semi-automatic brick press machine | 1 machine | 1,25,000 | 1,25,000 |
| Electric mixer + weighing system | 1 unit | 55,000 | 55,000 |
| Labour cost | - | - | 10,000 |
| Curing chamber setup (insulated) | Custom built | - | 40,000 |
| Safety gear and PPE Kits | 6 full kits | 2,000 per kit | 12,000 |
| Packaging and transport | High-quality bags ad demo tools | - | 15,000 |
| Marketing and branding | Posters, videos, flyers | - | 17,000 |
| Contingency and Miscellaneous | General expenses | - | 25,000 |

Total Estimated Budget =  ₹4,25,000

Cost of the one brick is 7 to 9 Rs

Conclusion for Geopolymer Cement bricks

Geopolymer cement bricks are a eco-friendly and strong option instead of using normal clay or cement bricks. They use waste materials like fly ash and GGBS, which help to cut down carbon emission and support circular economy ideas. If mixed and cured properly, these bricks can reach same or even better strength than normal bricks. This type of brick also support low-cost housing and better environment, so it can be used more in future buildings. It’s a good idea for large scale use and sustainable construction works.

References

- Alahmari T, Abdalla T, Rihan M. Review of recent developments regarding the durability performance of eco-friendly geopolymer concrete. Buildings. 2023;13(12):3033. https://doi.org/10.3390/buildings13123033.

- Asa E, Shrestha M, Baffoe-Twum E, Awuku B. Development of sustainable construction material from fly ash class c. J Eng Design Technol. 2020;18(6):1615–40. https://doi.org/10.1108/jedt-06-2019-0156.

- Dave N, Sahu V, Misra A. Development of geopolymer cement concrete for highway infrastructure applications. J Eng Design Technol. 2020;18(5):1321–33. https://doi.org/10.1108/jedt-10-2019-0263.

- McLellan BC, Williams RP, Lay J, Van Riessen A, Corder GD. Costs and carbon emissions for geopolymer pastes in compari son to ordinary portland cement. J Clean Prod. 2011;19(9–10):1080–90.

- Shaikh F, Vimonsatit V. Compressive strength of fly-ash-based geopolymer concrete at elevated temperatures. Fire Mater. 2014;39(2):174–88. https://doi.org/10.1002/fam.2240.

- Youssef, N.; Rabenantoandro, A.Z.; Dakhli, Z.; Hage Chehade, F.; Lafhaj, Z. Environmental evaluation of geopolymer bricks. In Proceedings of the MATEC Web of Conferences, INCER 2019, Beirut, Lebanon, 3–5 April 2019; pp. 1–5.

- Centrale Lille, UMR 9013–LaMcube–Laboratoire de Mécanique, Multiphysique, Multi-échelle, F-59000 Lille, France Economic Analysis of Geopolymer Brick Manufacturing: A French Case Study https://doi.org/10.3390/su12187403